TCHAR Block

Mathematical Derivation for Turbine

WHAMO Water Hammer Network Designer

Hydraulic Transient Analysis Software

Version 1.0  •  June 2026

# 1.  Introduction

The TCHAR (Turbine Characteristics) block is the central data structure used by the WHAMO engine to define the complete hydraulic and mechanical performance map of a hydraulic turbine during both steady-state and transient simulation.

WHAMO solves the full set of 1-D water-hammer equations (Method of Characteristics) while coupling, at each time step, to the turbine boundary condition defined by the TCHAR lookup tables. The TCHAR block stores dimensionless head, flow and efficiency data over a grid of gate openings and net heads, enabling the engine to interpolate instantaneous operating conditions throughout any transient event (load rejection, emergency shutdown, governor response, etc.).

# 2.  TCHAR Block Structure in .INP File

The complete TCHAR block syntax as written to the WHAMO .INP file is:

TCHAR  TYPE  <n>

GATE   <g1>  <g2>  ...  <gm>

HEAD   <h1>  <h2>  ...  <hk>

QMATRIX

<Q(g1,h1)>  <Q(g1,h2)>  ...  <Q(g1,hk)>

<Q(g2,h1)>  ...

...

EFFICIENCY

<η(g1,h1)>  <η(g1,h2)>  ...  <η(g1,hk)>

<η(g2,h1)>  ...

...

FINISH

Where:

n — Integer type identifier linking to a TURBINE block (TYPE keyword).

g1…gm — Gate (wicket-gate) opening positions in % (0–100). Defines m gate levels.

h1…hk — Net head values at which performance is tabulated (ft in FPS, m in SI).

QMATRIX — m × k matrix of turbine flow at each (gate, head) combination (cfs or m³/s).

EFFICIENCY — m × k matrix of turbine hydraulic efficiency η (dimensionless, 0–1).

# 3.  Complete Turbine Element Variables

The following table lists every variable associated with the TURBINE block and the TCHAR block in WHAMO Designer. All variables are required for a complete turbine definition.

| Variable | INP Keyword | Units (SI / FPS) | Description |
| --- | --- | --- | --- |
| label / ID | ID | — | Unique alphanumeric identifier for the turbine element. |
| turbineType | TYPE | Integer | TCHAR type number. Links turbine element to its TCHAR performance table. |
| syncSpeed | SYNCSPD | RPM | Synchronous (rated) rotational speed of the turbine-generator shaft. |
| wr2 | WR2 | kg·m² / lb·ft² | Flywheel effect (moment of inertia × g). Controls rotational inertia and speed transient response. WR² = I × g. |
| turbineDiameter | DIAMETER | m / ft | Runner (wheel) diameter. Optional; used internally for specific speed and scaling calculations. |
| turbFriction | FRICTION | — | Friction torque coefficient representing mechanical friction losses in the turbine-generator unit. |
| windage | WINDAGE | — | Windage (aerodynamic drag) torque coefficient acting on the rotating unit. |
| operationMode | OPTURB keyword | — | Operational mode: TURBINE (constant gate), GENERATE (power schedule), TURBGOV (governor-controlled), EMERGENCY (emergency shutdown). |
| vScheduleNumber | VSCHEDULE | Integer | Reference number of the VSCHEDULE block (gate vs. time table). Required when operationMode ≠ TURBINE. |
| pumpStatus | — | — | ACTIVE / INACTIVE flag. Controls whether an OPTURB block is written. |
| comment | c (line prefix) | — | Optional free-text comment written as a comment line above the block. |
| unit | — | SI / FPS | Unit system for this element. Triggers automatic unit conversion during INP export. |

# 4.  TCHAR Data Sub-Variables

The following variables are stored globally per TCHAR TYPE and are shared across all turbine elements of the same type:

| Sub-Variable | INP Keyword | Dimensions | Description |
| --- | --- | --- | --- |
| gate[ ] | GATE | 1 × m array | Gate opening percentages (%), e.g. [20, 40, 60, 80, 100]. Defines m wicket-gate positions. |
| head[ ] | HEAD | 1 × k array | Net head values (ft or m). Defines k head reference levels for the performance matrix. |
| qMatrix[ ][ ] | QMATRIX | m × k matrix | Turbine discharge at each (gate, head) pair. Q[i][j] = flow when gate = gate[i] and head = head[j]. |
| effMatrix[ ][ ] | EFFICIENCY | m × k matrix | Hydraulic efficiency at each (gate, head) pair. η[i][j] = efficiency (0–1) at gate[i], head[j]. |

# 5.  Mathematical Derivation

## 5.1  Equation of Motion for the Turbine-Generator Unit

During a hydraulic transient the turbine and generator act as a coupled rotating mass. The angular momentum equation (Newton's second law for rotation) governs the shaft speed N:

I · (dω/dt)  =  T_turbine(Q, H, G)  −  T_load  −  T_friction  −  T_windage

Where:

I — Mass moment of inertia of turbine + generator (kg·m² or slug·ft²). Note: WR² = I × g, so I = WR² / g.

ω — Angular velocity of the shaft (rad/s). ω = 2π N / 60.

T_turbine — Hydraulic torque developed by the runner (N·m or lb·ft). Function of instantaneous flow Q, net head H, and gate opening G.

T_load — Generator electromagnetic (load) torque (N·m or lb·ft). Set to zero in load-rejection scenarios.

T_friction — Mechanical friction torque = FRICTION × T_rated.

T_windage — Aerodynamic drag torque = WINDAGE × (N/N_sync)² × T_rated.

## 5.2  WR² and Moment of Inertia

WHAMO uses the traditional flywheel constant WR² (also written GD²/4 in SI for diameter-based systems). The relationship between WR², angular inertia I, and the time constant T_a is:

I  =  WR² / g

T_a  =  (WR² · N_sync²) / (375 · P_rated)        [FPS, P in horsepower]

T_a  =  (WR² · N_sync²) / (182.4 · P_rated)      [SI, P in kW, WR² in kg·m²]

T_a (seconds) is the mechanical starting time — the time to accelerate the unit from rest to rated speed at rated torque. Larger WR² leads to slower speed rise on load rejection and smoother transients.

## 5.3  Turbine Hydraulic Torque from TCHAR

The hydraulic torque delivered to the shaft is derived from power and speed:

P_turbine  =  γ · Q · H · η(Q, H, G)

T_turbine  =  P_turbine / ω  =  (γ · Q · H · η) / ω

Where:

γ — Specific weight of water (9810 N/m³ in SI, 62.4 lb/ft³ in FPS).

Q — Volumetric flow rate (m³/s or ft³/s) — interpolated from QMATRIX.

H — Net head across the turbine (m or ft).

η — Hydraulic efficiency (dimensionless) — interpolated from EFFICIENCY matrix.

ω — Shaft angular velocity (rad/s). At rated conditions ω = 2π · SYNCSPD / 60.

## 5.4  TCHAR Bilinear Interpolation

At every time step, WHAMO locates the turbine operating point (G, H) within the GATE × HEAD grid and performs bilinear interpolation to obtain Q and η:

f(G, H)  =  (1−s)(1−t)·f₀₀  +  s(1−t)·f₁₀  +  (1−s)t·f₀₁  +  st·f₁₁

Where s = (G − G_i)/(G_{i+1} − G_i) and t = (H − H_j)/(H_{j+1} − H_j) are normalised fractions. f represents Q or η at the surrounding grid nodes (i,j), (i+1,j), (i,j+1), (i+1,j+1).

## 5.5  Net Head Definition

The net head H used in the TCHAR lookup is the difference between the total energy at the turbine inlet and the energy at the draft-tube exit, accounting for penstock friction losses:

H_net  =  (p_in/γ + z_in + V_in²/2g)  −  (p_out/γ + z_out + V_out²/2g)  −  ΣhL

WHAMO computes H_net at each time step from the Method of Characteristics (MOC) solution at the turbine boundary node.

## 5.6  Speed-Rise on Load Rejection

When the generator breaker opens (T_load → 0), the net accelerating torque drives the turbine to runaway speed. The speed-rise equation integrated by WHAMO is:

N(t+Δt)  =  N(t)  +  Δt · (T_net / I)  ·  (60 / 2π)

Where T_net = T_turbine − T_friction − T_windage. The governor acts to close the wicket gates according to the VSCHEDULE gate vs. time programme, reducing T_turbine until equilibrium is reached or the unit shuts down.

## 5.7  VSCHEDULE (Gate-vs-Time) Coupling

When operationMode = GENERATE, TURBGOV, or EMERGENCY, the gate opening G at each time step is read from a VSCHEDULE table:

G(t)  =  linear_interpolation( T[ ], G[ ], t )

SCHEDULE

VSCHEDULE  1

T  0.0   G  100.0

T  2.0   G  100.0

T  5.0   G   50.0

T 10.0   G    0.0

FINISH

Linear interpolation between adjacent T/G pairs gives the instantaneous gate position G(t), which is then used in the TCHAR bilinear lookup.

## 5.8  Specific Speed and TCHAR Curve Selection

Turbine type curves are characterised by the dimensionless specific speed N_s. The three hill-chart families used in WHAMO Designer are:

| Specific Speed Ns (US) | Ns (SI) | SI Range | Typical Turbine Type |
| --- | --- | --- | --- |
| 30 | 115 | 111–178 | Francis – high head, low flow |
| 45 | 170 | 178–214 | Francis – medium head |
| 60 | 230 | 214–260 | Francis – low head, high flow |

N_s  =  N · P^(1/2)  /  H^(5/4)      [US: N rpm, P hp, H ft]

N_s  =  N · Q^(1/2)  /  H^(3/4)      [SI: N rpm, Q m³/s, H m]

## 5.9  Efficiency from Torque Coefficient

When the TCHAR table is auto-generated from a hill-chart library (torque-coefficient format), the efficiency η is derived as:

η  =  P_out / P_in  =  (T · ω) / (γ · Q · H)

The torque coefficient C_T relates turbine torque to head and runner diameter D:

T  =  C_T · ρ · n² · D⁵

Rearranging with the discharge coefficient C_Q = Q / (n · D³):

η  =  (C_T · ω) / (γ · C_Q · H · D²)

Efficiency values are always clamped to the range [0.0, 1.0] before being stored in the EFFICIENCY matrix.

## 5.10  Normalization and Scaling

Hill-chart library data is normalized at the design (best efficiency) point:

H_ref = 100,   Q_ref = 100   (dimensionless)

Actual values are obtained by applying design-point scaling factors:

Q_actual  =  Q_normalized · (Q_design / Q_ref)

H_actual  =  H_normalized · (H_design / H_ref)

# 6.  OPTURB Block and Operational Modes

Every turbine element requires a corresponding OPTURB block that sets the operational boundary condition for the transient simulation:

| Mode | INP Keyword | Description |
| --- | --- | --- |
| Turbine (constant gate) | TURBINE | Gate stays fixed. Turbine operates at initial steady-state gate opening throughout the simulation. |
| Generate (power schedule) | GENERATE | Gate follows the VSCHEDULE gate-time programme while the turbine generates power. |
| Governor-controlled | TURBGOV | Gate is controlled by a governor model responding to speed deviation. Gate programme via VSCHEDULE. |
| Emergency shutdown | EMERGENCY | Rapid gate closure on trip event. Emergency closure profile defined in VSCHEDULE. |

Example OPTURB block (TURBGOV mode):

OPTURB

ID       TU01

TURBGOV

VSCHEDULE  1

FINISH

# 7.  Complete Sample INP Block

Below is a fully annotated sample TURBINE + TCHAR + OPTURB + VSCHEDULE block for a single Francis turbine:

c  Francis Turbine — Unit TU01  (Ns=30, Design Head=100 ft)

TURBINE

ID         TU01

TYPE       1

SYNCSPD    600.

WR2        125000.

DIAMETER   9.5

FRICTION   0.02

WINDAGE    0.005

FINISH

TCHAR  TYPE  1

GATE  20.  40.  60.  80.  100.

HEAD  80.  90.  100.  110.  120.

QMATRIX

48.4  52.0  55.0  57.8  60.1

77.4  82.5  87.5  92.0  96.0

113.1 120.5 128.0 135.0 141.0

150.3 160.5 170.5 180.0 188.0

187.5 200.0 212.0 222.0 230.0

EFFICIENCY

0.60  0.63  0.65  0.66  0.66

0.72  0.76  0.78  0.79  0.79

0.80  0.85  0.88  0.89  0.89

0.84  0.88  0.91  0.92  0.92

0.86  0.90  0.92  0.93  0.93

FINISH

OPTURB

ID    TU01

GENERATE

VSCHEDULE  1

FINISH

SCHEDULE

VSCHEDULE  1

T   0.0   G  100.0

T   2.0   G  100.0

T   5.0   G   60.0

T  10.0   G    0.0

FINISH

# 8.  Full Variable Reference Summary

| Variable | Type | Default | Units | Notes |
| --- | --- | --- | --- | --- |
| ID / label | String | Required | — | Max 12 chars. Must be unique within project. |
| turbineType (TYPE) | Integer | 1 | — | Links to TCHAR TYPE n. Multiple turbines may share one TCHAR. |
| syncSpeed (SYNCSPD) | Float | 0.0 | RPM | Rated synchronous speed. e.g. 600 rpm (10-pole, 60 Hz). |
| wr2 (WR2) | Float | 0.0 | lb·ft² / kg·m² | Flywheel effect. Higher WR² → slower speed rise → smoother transients. |
| turbineDiameter (DIAMETER) | Float | Optional | ft / m | Runner diameter. Optional field; omitted if 0. |
| turbFriction (FRICTION) | Float | 0.0 | — | Fraction of rated torque lost to friction. |
| windage (WINDAGE) | Float | 0.0 | — | Fraction of rated torque lost to windage (speed-squared dependent). |
| operationMode | Enum | TURBINE | — | TURBINE ¦ GENERATE ¦ TURBGOV ¦ EMERGENCY |
| vScheduleNumber | Integer | 1 | — | VSCHEDULE reference. Used only with GENERATE/TURBGOV/EMERGENCY. |
| GATE[ ] | Float[ ] | [] | % | Gate positions 0–100%. Array of m values. |
| HEAD[ ] | Float[ ] | [] | ft/m | Net head array. k reference head levels. |
| QMATRIX[ ][ ] | Float[][] | [] | ft³/s / m³/s | Flow matrix m×k. Row=gate, Col=head. |
| EFFICIENCY[ ][ ] | Float[][] | [] | dimensionless | Efficiency matrix m×k. Values 0–1. |
| V-Schedule T[ ] | Float[ ] | [] | seconds | Time points for gate-position schedule. |
| V-Schedule G[ ] | Float[ ] | [] | % | Gate position at each time point (0–100%). |
| unit | Enum | SI | — | SI or FPS. Controls automatic unit conversion on export. |
| comment | String | Optional | — | Prefixed with 'c ' as INP comment line. |